System Overview

The smart bed automation system is designed to assist patients and elderly persons by providing gesture-based bed movement control, health monitoring, medicine dispensing, and emergency notifications. The system utilizes Arduino Uno as the main controller, interfaced with various sensors and actuators to achieve the desired functionalities.

Working Description

1. Hand Gesture Recognition Using ADXL335 Accelerometer

- The ADXL335 accelerometer is used to recognize hand gestures to control the movement of the bed.

- The accelerometer measures the acceleration forces in the x, y, and z axes.

- Based on predefined gesture thresholds (tilt left/right for bed tilt, up/down for bed height adjustment), the Arduino processes the accelerometer data to recognize gestures.

- When a gesture is detected, corresponding signals are sent to the motor drivers to move the bed accordingly.

2. Bed Movement Control Using DC Motors

- The bed movement is controlled using DC motors connected to motor drivers (e.g., L298N).

- Two types of bed movements are controlled: tilt and height adjustment.

- Based on the recognized gestures, the Arduino sends PWM signals to the motor drivers to control the speed and direction of the DC motors, enabling smooth and precise bed movement.

3. Temperature Monitoring Using DS18B20 Sensor

- The DS18B20 temperature sensor is used to monitor the patient's body temperature.

- The sensor communicates with the Arduino using the 1-Wire protocol, providing accurate temperature readings.

- The Arduino continuously reads the temperature data and displays it on the LCD screen.

- If the temperature exceeds a predefined threshold, an emergency notification is sent to the caretaker via Telegram.

4. Heartbeat Monitoring Using MAX30100 Sensor

- The MAX30100 sensor is used to monitor the patient's heart rate and oxygen levels.

- The sensor combines a pulse oximeter and a heart rate monitor, providing real-time data.

- The Arduino processes the sensor data to calculate the heart rate and displays it on the LCD screen.

- If the heart rate falls outside the normal range, an emergency notification is sent to the caretaker via Telegram.

5. Medicine Dispensing Box Using DC Motor

- A DC motor is used to automate the dispensing of medicines from a box.

- The medicine dispensing schedule is programmed into the Arduino.

- At the scheduled times, the Arduino activates the DC motor to dispense the required dosage of medicine.

- The LCD display shows a notification indicating that the medicine has been dispensed.

6. LCD Display for Process Display

- A 16x2 LCD display is used to show real-time information and system status.

- The display shows temperature, heart rate, bed movement status, and medicine dispensing notifications.

- The Arduino controls the LCD using the I2C protocol for efficient data communication.

7. Emergency Notifications Using NodeMCU and Telegram

- The NodeMCU (ESP8266) is used for Wi-Fi connectivity and sending notifications via Telegram.

- The Arduino communicates with the NodeMCU using serial communication.

- When an emergency condition is detected (e.g., abnormal temperature or heart rate), the Arduino sends a signal to the NodeMCU.

- The NodeMCU then sends a preconfigured message to the caretaker's Telegram account, alerting them of the emergency.

This smart bed automation system provides a comprehensive solution for assisting patients and elderly persons. By integrating gesture-based bed control, health monitoring, automated medicine dispensing, and emergency notifications, it enhances the quality of care and ensures timely assistance in case of health emergencies. The system leverages the capabilities of Arduino and NodeMCU to create an efficient and reliable automation solution.

Component Details:

Arduino Microcontroller:

The Arduino microcontroller acts as the central control unit for the smart bed automation system. It receives input from various sensors, processes the data, and controls the actuators accordingly.

2. Accelerometer (ADXL335) for Hand Gesture Recognition:

- Purpose: The ADXL335 accelerometer sensor is used to detect hand gestures made by the user.

- Functionality: The sensor measures the acceleration in three axes (X, Y, and Z). These measurements are interpreted to recognize specific hand gestures.

- Integration: The sensor is connected to the Arduino, which processes the accelerometer data to detect gestures and trigger corresponding actions, such as adjusting the bed position.

3. DC Motors for Bed Movement Control:

- Purpose: DC motors are used to control the movement of the bed, allowing it to tilt, raise, or lower as needed.

- Functionality: The motors are controlled by the Arduino based on the hand gestures detected by the accelerometer. Motor drivers are used to interface the motors with the Arduino.

- Integration: The Arduino sends signals to the motor drivers to adjust the bed position according to user gestures.

4. Temperature Sensor (DS18B20):

- Purpose: The DS18B20 temperature sensor is used to monitor the temperature of the patient or elderly person.

- Functionality: The sensor provides digital temperature readings to the Arduino.

- Integration: The Arduino continuously reads the temperature data and can trigger an alert if the temperature goes beyond a predefined threshold.

5. Heart Beat Sensor (MAX30100):

- Purpose: The MAX30100 sensor is used to monitor the heart rate of the user.

- Functionality: The sensor combines a pulse oximeter and heart rate monitor. It measures the oxygen saturation and pulse rate.

- Integration: The Arduino processes the heart rate data and can send an alert if the heart rate deviates from normal levels.

6. DC Motor-Based Medicine Dispensing Box:

- Purpose: The medicine dispensing box is designed to dispense medication at predefined times.

- Functionality: Controlled by a DC motor, the box releases medicine doses as programmed.

- Integration: The Arduino controls the motor based on a schedule or user input to ensure timely medication dispensing.

7. 2*16 LCD for Process Display:

- Purpose: The LCD display provides real-time information about the system status and processes.

- Functionality: The display shows data such as current temperature, heart rate, bed position, and medication dispensing status.

- Integration: The Arduino updates the LCD with relevant information, keeping the user informed about the system operations.

8. NodeMCU for Telegram Notifications:

- Purpose: The NodeMCU is used to send notifications to a caretaker via Telegram in case of a health emergency.

- Functionality: The NodeMCU connects to Wi-Fi and uses the Telegram API to send messages.

- Integration: The Arduino communicates with the NodeMCU to trigger notifications when an emergency is detected, such as abnormal temperature or heart rate readings.

System Workflow

1. Hand Gesture Recognition:

- The ADXL335 accelerometer detects hand gestures.

- The Arduino processes these gestures to determine the desired bed movement.

- The Arduino sends signals to the DC motors to adjust the bed position accordingly.

2. Health Monitoring:

- The DS18B20 sensor continuously monitors the patient's temperature.

- The MAX30100 sensor monitors the heart rate and oxygen saturation.

- The Arduino processes the data from these sensors and updates the 2*16 LCD display with real-time health information.

- If the temperature or heart rate exceeds normal levels, the Arduino triggers an emergency alert.

3. Medicine Dispensing:

- The Arduino controls the DC motor-based medicine dispensing box to release medication at scheduled times.

- The status of medicine dispensing is displayed on the LCD.

4. Emergency Notifications:

- In case of a health emergency, the Arduino communicates with the NodeMCU.

- The NodeMCU sends a Telegram notification to the caretaker, providing details of the emergency.

The smart bed automation system integrates various sensors and actuators to provide a comprehensive solution for patient and elderly care. By using Arduino as the central controller, the system ensures real-time monitoring and control, enhancing the safety and comfort of the user. The inclusion of hand gesture recognition, health monitoring, medicine dispensing, and emergency notifications makes it a versatile and effective solution for healthcare needs.